First、Install EPSON ADK

1. Unzip the EPSON ADK installer, enter the Disk1 directory, and double-click to run the "setup.exe" installation file.

2. Select the language and click OK to go to the next step

3, setup Welcome page, click "Next" to go to the next step

4. Software license Agreement, check "I accept the terms of the license agreement" and click "Next" to get to the next step

5 . Select the registry file ( with the registry backup file to select " Use registry file " and select the registry backup file correctly . No " No registry file " ) , and click Next to enter the next step

6. Select the installation type, with developer user Developer to choose from, and choose "Developer" according to your needs. Click "Next"

7, confirm the settings before installation, need to change settings click "Back" return, do not need to change click "Next"

8, select the port, select "Serial port", and click "Next" to go to the next step

9, the program starts to install, please wait patiently

10 . Installation is complete , " Display the release notes " is not checked , and click Finish to complete the installation .

Second、Driver profile replacement

1. Copy the LdDMD110.inf file we provided

2, go to the EPSON ADK program installation directory, for example: "C:\ Program Files x86)\ OPOS\ Epson2". Paste the LdDMD110.inf file into this directory and replace the original file.

Third、Add LineDisplay devices

1. Go to the "start" menu-"all programs"-"OPOS"-click "SetupPOS Ver2.00". Open "EPSON OPOS ADK Version2.80E UPOS 1.14"

2. Find "LineDisplay" in the right device list, right-click the pop-up menu, and click "Add New Device"

3, keep the default settings, and click "Next" to go to the next

4. Leave the default setting for "Stand alone" and click OK to go to the next step

5, according to the actual situation select serial port number, baud rate, others to maintain the default settings, click on "Finish" to complete the settings.

Note: if the "Handshake" option is not the default "DISABLE" and the drop-down list does not have the "DISABLE" option. The driver profile may not have been executed or replaced successfully. Try adding the device after following the instructions for driver profile replacement in this article

Fourth、OPOS LindeDisplay test

1 . Open the OposTest . exe test program

2, test operation order: Open-Claim-Device enabled, Display Text. Clear Text.